SECTION 07 46 16

ARCHITECTURAL ALUMINUM PLANK CLADDING

PART 1 – GENERAL

1.01 SUMMARY

A. Section Includes:

- Extruded aluminum plank wall and soffit cladding system.

- Extruded aluminum trims and accessories.

- Factory-finished aluminum plank components.

- Concealed fastening attachment system.

- Factory-finished corners, transitions, and splice components.

B. Related Sections:

- Section 05 40 00 – Cold-Formed Metal Framing.

- Section 06 10 00 – Rough Carpentry

- Section 07 21 00 – Thermal Insulation.

- Section 07 27 00 – Air Barriers.

- Section 07 62 00 – Sheet Metal Flashing and Trim.

- Section 07 84 00 – Firestopping.

- Section 07 92 00 – Joint Sealants

1.02 REFERENCES

1. American Society for Testing and Materials (ASTM)

- ASTM C423 - Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method (NRC)

- ASTM E 84 Standard Test Method for Surface Burning Characteristics of Building Materials

- ASTM E136 - Standard Test Method for Behavior of Materials in a Vertical Tube Furnace at 750°C

- ASTM E283-04 - Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen

- ASTM E331-00 - Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Uniform Static Air Pressure Difference

- ASTM E1477 - Standard Test Method for Luminous Reflectance Factor of Acoustical Materials by Use of Integrating-Sphere Reflectometers (LRV)

- ASTM E2768-11 – Standard Test Method for Extended Duration Surface Burning Characteristics for Building Materials (30 min Tunnel Test). Results: Zero Flame Spread, Smoke Developed Index of 5. Meets criteria for Class A fire rating

1.03 PERFORMANCE REQUIREMENTS

- Structural Performance:

  - System shall withstand positive and negative design wind pressures as indicated on project drawings.

  - System shall be tested in accordance with ASTM E330.

  - Plank system shall demonstrate performance up to 180 psf Ultimate Wind Load utilizing fastener spacing up to 48 inches on center.

- Thermal Movement:

  - System shall accommodate expansion and contraction resulting from surface temperature changes without damage to components or reduction in performance.

- Fire Performance:

  - Aluminum plank components shall be non-combustible.

1.04 SUBMITTALS

- Product data: submit manufacturer's printed product literature, specifications and data sheet.

- Submit 4 inch X 8 inch samples of cladding material, of color and profile specified.

- Shop drawings to indicate dimensions, profiles, attachment methods, schedule of wall elevations, trim and closure pieces, soffits, fascia, metal furring, and related work.

- Certifications: Manufacturer's certifications that products comply with specified requirements, including laboratory reports showing compliance with specified tests and standards.

- LEED Submittal Data: Manufacturer’s product data for each product specified in this section per ecoscorecard.com.

- Submit manufacturer’s installation instructions.

1.05 QUALITY ASSURANCE

- Manufacturer Qualifications:

  - Manufacturer shall have a minimum of ten (10) years experience producing architectural metal cladding systems.

  - Manufacturer shall maintain an established manufacturing facility capable of producing project-specific components.

- Installer Qualifications:

  - Installer shall have demonstrated experience installing architectural metal cladding systems of similar size and complexity.

  - WARRANTY

- Manufacturer Warranty:

  - Two (2) year manufacturer warranty covering defects in material and workmanship.

- Finish Warranty:

  - Powder Coating: Up to 25 years depending on coating system selected.

  - Woodgrain Sublimation Finish: Up to 25 years depending on finish selection and location of the project.

PART 2 – PRODUCTS

2.01 MANUFACTURER

    - Basis of Design:

Zeeff Panel Systems

Zeeland, Michigan

[www.zeeffpanels.com]

- Approved Equal:

  - Approved equal meeting all specified requirements.

  - ALUMINUM PLANK SYSTEM

- Product:

  - ZEEFF PANEL SYSTEMS

    - LINEAR Classic Architectural Aluminum Plank System

- Description:

  - Extruded aluminum plank cladding system for exterior wall and soffit applications.

  - Concealed-fastened system utilizing integrated Z-Lock engagement profile.

  - Suitable for horizontal and vertical installation orientations.

- Material:

  - Extruded Aluminum Alloy: 6063-T6.

  - Nominal Thickness: 0.075 inch.

  - Non-combustible construction.

- Dimensions:

  - Nominal Widths:

    - 6 inches.

    - 8 inches.

- Standard Depth:

  - 1/2 inch.

- Lengths:

  - Lengths up to 16 feet.

- Reveals:

  - Smooth

  - V-Groove Reveal.

- Attachment:

  - Integrated concealed fastener attachment system.

  - Separate attachment clips are not acceptable.

  - Fastener spacing as required by project engineering and manufacturer testing.

- Accessories:

  - Base Extrusions.

  - H Extrusions.

  - Jamb Extrusions.

  - Outside Corner Extrusions.

  - Integrated Color-Matched Splice Plates.

  - Closure Components.

  - Flashings and Trim.

2.03 FINISHES

- Powder Coating:

  - High-performance FEVE or Super Durable Powder Coating.

  - AAMA 2604 compliant finishes.

  - AMA 2605 compliant finishes available.

- Woodgrain Finishes:

  - Architectural sublimation woodgrain finish.

  - AAMA 2604 compliant performance.

  - Standard manufacturer woodgrain selections.

  - Custom woodgrain finishes available.

- Color Selection:

  - Manufacturer's standard colors.

  - Premium colors.

  - Custom color matching available.

2.04 FASTENERS

- Corrosion-resistant fasteners compatible with aluminum construction.

- Fasteners shall be concealed whenever possible.

- Fastener type shall be selected based on substrate conditions and project-specific engineering requirements.

PART 3 – EXECUTION

3.01 EXAMINATION

- Verify substrates are plumb, level, structurally sound, and ready to receive work.

- Do not proceed until unsatisfactory conditions have been corrected.

3.02 INSTALLATION

- Install planks in accordance with manufacturer’s written installation instructions and approved shop drawings.

- Install components true to line, level, and plane.

- Maintain consistent reveal spacing.

- Allow for thermal movement.

- Protect finished surfaces during installation.

3.03 FIELD QUALITY CONTROL

- Verify alignment, reveal consistency, and secure attachment.

- Remove damaged materials and replace with new materials.

3.04 CLEANING

- Remove debris and protective coverings.

- Clean finished surfaces using mild detergent, water, and following manufacturer's cleaning instructions.

- Do not use abrasive cleaners, acidic cleaners, steel wool, or aggressive solvents.

END OF SECTION 07 42 13